Name:  Momna Ashraf                                                    Registeration No: 2025-CE-72

Date:   11-05-2026                                                          Grade and Signature:  ............

LAB PROCEDURE – Verification of Thevenin’s Theorem

- Connect the circuit according to the given circuit diagram on the breadboard or trainer board.

- Check all circuit connections carefully before switching on the power supply.

- Remove the load resistance  from the circuit and keep the load terminals open.

- Switch on the DC power supply and measure the open-circuit voltage across the load terminals using a voltmeter. Record this value as the Thevenin voltage

- Turn off the power supply and remove all independent sources from the circuit.

  - Replace voltage sources with short circuits.

  - Replace current sources with open circuits.

- Measure the equivalent resistance across the load terminals using a multimeter. Record this value as the Thevenin resistance.

- Draw the Thevenin equivalent circuit using the measured values of Vth  and R

- Reconnect the load resistance   to the Thevenin equivalent circuit.

- Measure the load current and load voltage in the equivalent circuit.

- Compare the measured values with the values obtained from the original circuit to verify Thevenin’s Theorem.

- Record all observations and calculations in the observation table.

- Conclude whether Thevenin’s Theorem is verified successfully or not.

# Explanation

Thevenin’s Theorem is used to simplify a complex electrical network into an equivalent circuit containing only one voltage source and one series resistance .
 In this experiment, the open-circuit voltage across the load terminals is measured to determine the Thevenin voltage. Then, the equivalent resistance of the circuit is calculated or measured after removing all independent sources.

After obtaining  the original circuit is replaced with its Thevenin equivalent circuit. The load resistance is connected again, and the load current and voltage are measured. If these values match the values obtained from the original circuit, then Thevenin’s Theorem is verified successfully.

This theorem helps in reducing complicated circuit analysis and is widely used in electrical and electronic engineering applications.

# Precautions

- Ensure all circuit connections are correct and tight before turning on the power supply.

- Switch off the power supply before changing any circuit connection.

- Use properly calibrated measuring instruments for accurate readings.

- Remove the load resistance carefully while measuring open-circuit voltage.

- While calculating , replace voltage sources with short circuits and current sources with open circuits correctly.

- Avoid short-circuiting the power supply accidentally.

- Take readings carefully to minimize human and instrumental errors.

- Do not exceed the rated voltage and current values of the circuit components.

- Verify multimeter settings before measuring voltage or resistance.

- Record all observations neatly and accurately in the observation table.