Submitted By:

Momna Ashraf – 2025-CE-72

Assignment:

Lab No. 1

Submitted To:

Sir Mehmood

For the fulfillment of,

Circuit Analysis Lab

Department of Computer Engineering

University of Engineering and Technology, Lahore

POST LAB QUESTIONS:

1. Define Oscilloscope and write its applications.

An oscilloscope is an electronic measuring instrument used to display and analyze electrical signals in the form of waveforms. It shows how voltage changes with respect to time on a graphical screen, allowing engineers and students to observe signal behavior.

Applications of Oscilloscope:

- To measure voltage of electrical signals.

- To observe waveform shapes such as sine, square, and triangular waves.

- To measure frequency and time period of signals.

- To analyze electronic circuits and troubleshoot faults.

- To compare input and output signals in electronic systems.

2. What is Peak to Peak Voltage?

Peak-to-Peak Voltage (Vpp) is the total voltage difference between the maximum positive peak and the maximum negative peak of a waveform. It represents the full vertical height of the signal on an oscilloscope screen.

Mathematically:

Vpp = Vmax − Vmin

For symmetrical waves, the peak-to-peak voltage is equal to twice the peak voltage.

3. What is RMS Value?

RMS (Root Mean Square) value is the effective value of an alternating voltage or current. It represents the amount of DC voltage that would produce the same power in a resistor as the AC voltage.

It is calculated by taking the square root of the average of the squares of the instantaneous values of the signal over one complete cycle. RMS value is commonly used because it represents the real usable power of an AC signal.

4. Write the RMS values of Sine, Square and Triangular Waves.

The RMS values of common waveforms are:

- Sine Wave:
 Vrms = Vpeak / √2
 (approximately 0.707 × Vpeak)

- Square Wave:
 Vrms = Vpeak

- Triangular Wave:
 Vrms = Vpeak / √3
 (approximately 0.577 × Vpeak)

5. Explain DC Offset.

DC Offset refers to the amount by which a waveform is shifted above or below the zero-voltage reference level. It occurs when a constant DC voltage is added to an AC signal.

If the DC offset is positive, the waveform shifts upward; if it is negative, the waveform shifts downward. DC offset is important in electronic circuits because it can affect signal measurements and circuit performance.