## Circuit Analysis (Lab # 6)

Experiment 6: VERIFICATION OF KIRCHHOFFS LAWS

#### Name:  Momna Ashraf.		Registration No: 2025-CE-72   Date: 25-4-2026	Grade and Signature: ………………………

| CLO2 CLO3 | Use with identification the circuit components, breadboards, multimeters, power supplies, signal generators, and oscilloscopes. Make measurements in their own constructed electric circuits. |  |  |  |
| --- | --- | --- | --- | --- |
| Psychomotor/Affective | Level1 (1) | Level2 (2-3) | Level3 (4-5) | Level4 (6-7) |
| Report Marks (3) |  |  | Total marks (10) |  |

# Objective

To verify Kirchhoff’s current law and Kirchhoff’s voltage law for the given Circuit.

Apparatus Required

| Sr. no | Apparatus | Range | Quantity |
| --- | --- | --- | --- |
| 1 | Regulated Power Supply | Standard | 1 |
| 2 | Signal Generator | Standard | 1 |
| 3 | Oscilloscope | Standard | 1 |
| 4 | Digital Multimeter | Standard | 1 |
| 5 | Breadboard & Wires | Standard | As Required |
| 6 | Resistors | 330ohm, 220ohm 1kohm | 3 |

### Description

KCL: The algebraic sum of the currents meeting at a node is equal to zero.

KVL: In any closed path / mesh, the algebraic sum of all the voltages is zero.

### Precautions:

- Voltage control knob should be kept at minimum position.

- Current control knob of RPS should be kept at maximum position.

### Procedure for KCL:

- First, the required components such as resistors, regulated power supply (RPS), ammeters (or multimeter), and connecting wires were arranged.

- The circuit was connected carefully according to the given circuit diagram. All connections were checked to avoid any loose contact.

- The ammeters were connected in series in different branches to measure the currents:

  - I₁ as the main current entering the junction

  - I₂ and I₃ as the currents leaving the junction

- The regulated power supply was switched on and set to 5V.

- The readings of I₁, I₂, and I₃ were noted using the ammeter.

- The relation I₁ = I₂ + I₃ was checked to verify Kirchhoff’s Current Law.

- The above steps were repeated for different values of voltage (10V and 15V).

- All the readings were recorded in the observation table.

Explanation:

Kirchhoff’s Current Law (KCL) states that the total current entering a junction is equal to the total current leaving the junction. This is based on the conservation of charge.

In this experiment, current from the power supply reaches a junction and then divides into two branches. The current splits depending on the resistance of each branch. It was observed that the sum of currents in the outgoing branches (I₂ + I₃) is approximately equal to the incoming current (I₁).

Any small difference in readings is due to experimental errors such as instrument inaccuracy, resistance tolerance, and loose connection

### Circuit - KCL

### KCL - Practical Values:

| Sl. No. | Voltage E | Current |  |  | I1 = I2 + I3 |
| --- | --- | --- | --- | --- | --- |
|  |  | I1 | I2 | I3 |  |
|  | Volts | mA | mA | mA | mA |
| 1 | 5 | 5.68 | 3.12 | 2.56 | 5.68 |
| 2 | 15 | 17.1 | 9.37 | 7.68 | 17.05 |
| 3 | 25 | 28.4 | 15.6 | 12.8 | 28.4 |

### Procedure for KVL:

- First, all the required components such as resistors (330Ω, 1kΩ, 220Ω), regulated power supply (RPS), voltmeters, and connecting wires were taken.

- The circuit was connected carefully according to the given circuit diagram. Proper care was taken to ensure that all connections were tight and correct.

- Voltmeters were connected in parallel across each resistor to measure the voltage drops:

  - V₁ across 330Ω resistor

  - V₂ across 1kΩ resistor

  - V₃ across 220Ω resistor

- The regulated power supply was switched on and set to 5V.

- The readings of V₁, V₂, and V₃ were noted using the voltmeters.

- The sum of voltage drops was calculated and compared with the source voltage to verify KVL.

- The above steps were repeated for different values of voltage (10V and 15V).

- All the readings were recorded in the observation table.

Explanation:

Kirchhoff’s Voltage Law (KVL) states that the sum of all voltages in a closed loop is equal to zero. This means that the total voltage supplied by the source is equal to the total voltage drop across the resistors in the circuit.

In this experiment, the voltage from the power supply is divided among the resistors. The 330Ω resistor is in series, while the 1kΩ and 220Ω resistors are connected in parallel. Since the resistors in parallel have the same voltage across them, V₂ and V₃ are equal.

When the readings were taken, it was observed that the sum of voltage drops across the resistors was approximately equal to the applied voltage. This verifies Kirchhoff’s Voltage Law.

Any small difference in values is due to experimental errors such as resistance tolerance, instrument error, and loose connections.

### Circuit - KVL

### KVL - Practical Values

| Sl. No. | RPS | Voltage |  |  | E1 = V1 + V2 |
| --- | --- | --- | --- | --- | --- |
|  | E1 | V1 | V2 | V3 |  |
|  | V | V | V | V | V |
| 1 | 5 | 3.23 | 1.77 | -1.77 | 5 |
| 2 | 10 | 6.47 | 3.53 | -3.53 | 10 |
| 3 | 15 | 9.76 | 5.30 | -5.30 | 15 |

### Result:

Thus, Kirchhoff’s voltage law and Kirchhoff’s current law are verified practically by performing this experiment.

Post Lab Questions – Answers

1. Define Kirchhoff’s Current Law (KCL):
 Kirchhoff’s Current Law (KCL) states that the total current entering a junction (or node) is equal to the total current leaving the junction. In other words, the algebraic sum of all currents at a node is zero. This law is based on the principle of conservation of charge, meaning charge cannot be created or destroyed at a node.

2. Define Kirchhoff’s Voltage Law (KVL):
 Kirchhoff’s Voltage Law (KVL) states that the sum of all voltages in any closed loop of a circuit is equal to zero. This implies that the total energy supplied to the circuit is equal to the total energy consumed within the loop. It is based on the principle of conservation of energy.

3. What are the applications of regulated power supply?
 A regulated power supply is used to provide a constant output voltage despite variations in input voltage or load conditions. It is commonly used in electronic devices such as computers, communication systems, laboratory instruments, and sensitive circuits where stable voltage is required for accurate and reliable operation.

4. What are the limitations of KCL?
 Kirchhoff’s Current Law assumes that no charge accumulates at a node, which may not hold true in high-frequency circuits. It does not consider displacement current in capacitive or electromagnetic fields. Therefore, in circuits involving high-frequency signals or complex electromagnetic effects, KCL may not give completely accurate results.