### CIRCUIT ANALYSIS – LAB 7

### Name:  Momna Ashraf

Reg.no: 2025-CE-72

### Explanation

An operational amplifier (op-amp) is used to amplify voltage signals. In this experiment, two configurations are studied: inverting and non-inverting amplifiers.

In an inverting amplifier, the input signal is applied to the inverting terminal, while the non-inverting terminal is grounded. The output signal is amplified and inverted, meaning it is 180° out of phase with the input. The voltage gain is given by:

Av = − (Rf / Rin)

In a non-inverting amplifier, the input signal is applied to the non-inverting terminal. The output signal is amplified but remains in phase with the input. The voltage gain is:

Av = 1 + (Rf / Rin)

These relationships show how resistor values control the amplification in both circuits.

### Procedure

- Connect the IC 741 op-amp to a dual power supply (±12V or ±15V).

- Set up the inverting amplifier circuit on the breadboard:

  - Apply input signal through resistor (Rin) to the inverting terminal.

  - Connect feedback resistor (Rf) between output and inverting terminal.

  - Ground the non-inverting terminal.

- Apply a sinusoidal input signal using the signal generator.

- Observe the input and output waveforms on the oscilloscope.

- Measure and note the input and output voltages.

- Set up the non-inverting amplifier circuit:

  - Apply input signal to the non-inverting terminal.

  - Connect resistors (Rf and Rin) in feedback configuration.

- Observe and record the waveforms and voltages for this circuit as well.

### Result

The inverting amplifier produced an output that was amplified and 180° out of phase with the input signal. The non-inverting amplifier produced an amplified output that remained in phase with the input. The observed behavior of both circuits was consistent with theoretical expectations.